For DNA minigel:

50 ml TE buffer with 1-2% agarose.

Microwave 1 minutes at half power.

Allow to cool a few minutes and add 2 ul 50 mg/ml ethidium bromide.

Pour into tray and let solidify for 10-15 minutes.

Run samples and standard at ~100-150 v until dye markers are clearly separated.